# A Brief Guide to SPC File Format and

# Using GSPCIO

Copyright © 2001. Thermo Galactic. All rights reserved.

395 Main Street  Salem, NH 03079  USA (603) 898-7600   www.thermogalactic.com

NOTE: This document describes the "new" SPC file format. The "old" format that was used in SpectraCalc and LabCalc does not support multifiles or the Audit Log and has a 256 byte Main header. However, GRAMS and the file I/O utilities must be capable of reading both formats.

A more detailed description of the SPC file format is available in the document "Galactic Universal Data Format Specification." This document provides a guide for programmers to implement data reading or data writing routines for files in Thermo Galactic SPC data file format. The "official" description of the SPC file format is contained in the file SPC.H. These documents are included in the Thermo Galactic SPC File Developer's Kit which can be downloaded from the Thermo Galactic website (www.thermogalactic.com/instruments/spcdata/spc_sdk.zip).

Table of Contents

SPC File Format	1

Single Files and Multifiles	1

SPC File Sections	1

Main Header Block	2

Data Block	3

XY Data Block	3

Subheader Block	3

Directory Block	4

Log Block	4

Log Header Block	4

Log Data Block	5

ASCII Log Text Block	5

Using GSPCIO.	6

Defining GSPCIO for Use in Programs	6

Reading File Properties	6

File Type	6

X and Y Values	6

X Axis Limits	7

Axis Labels	7

Subfiles	7

Audit Log	7

Creating SPC Files	8

Y-only Single Files	8

XY Single Files	8

Y-Only Multifiles	9

XYY Multifiles	9

XYXY Multifiles	10

Disk Files	10

Reading from Disk	10

Writing to Disk	11

© Thermo Galactic SPC File Format and Using GspcIO

# SPC File Format

The SPC file is the standard Thermo Galactic file format. This is a binary file containing all of the information necessary for displaying and processing spectral data in GRAMS and related applications.

## Single Files and Multifiles

The SPC file format allows the storage of either one or multiple spectra, plus associated data. An SPC file can be either a single file or a multifile. A Single file is a file containing just one spectrum. A Multifile is a file containing more than one spectrum. Each individual spectrum in a Multifile is referred to as a subfile.

A spectrum file can be viewed as an array where each Y point has a corresponding X value. It can either be a Y-only or an XY file.

The Y-only file is the standard format. Y-only files have X values that are equally spaced. A Y- only file stores only the Y data, with the starting and ending X-axis limits stored in the header. (With this information, GRAMS and other Thermo Galactic products can calculate the X value of any data point in the file "on-the-fly.") The X-axis limits are called the Frequency First Point (ffp) and Frequency Last Point (flp).

The XY file has X values that are not equally spaced. In this case, the X and Y values are both stored (two arrays of data points, one array for the Y values and a second array for the corresponding X values). The ffp/flp limits are ignored.

A multifile is simply a file with more than one spectrum (subfile) stored in it. There are three basic types of multifiles:

Y-Only All of the subfiles have the same evenly spaced X values. Only the Y values are stored.

XYY	All of the subfiles have the same unevenly spaced X values. The X values are stored once and the Y values are stored for each subfile.

XYXY	Each subfile has its own separate X and Y arrays.

## SPC File Sections

The SPC data format itself is broken down into three main categories (blocks) of information and it is important for developers to have a basic understanding of them in order to make the best use of the GSPCIO library.

In the order that they appear in the file format, these are the Main Header Block, the Data Block, and the Log Block.

### Main Header Block

A 512 byte Main Header contains information defining the global file parameters and has the following fields:

| Byte # | Data Type | Description |
| --- | --- | --- |
| 0 | byte | File type flag. Each bit contains a different parameter: 0 = Y data is stored in 16-bit precision (instead of 32-bit) 2 = Use Experiment extension, not SPC 4 = Multifile 8 = If a Multifile, Z values are randomly ordered 16 = If a Multifile, Z values are ordered, but not even 32 = Use custom axis labels (obsolete) 64 = If an XY file and a Multifile, each subfile has its own X array 128 = XY file |
| 1 | byte | SPC file version: 4B = New format 4D = Old LabCalc format |
| 2 | byte | Experiment type code (see SPC.H for values) |
| 3 | char | Exponent for Y values (80h = as floating point): FloatY = (2^Exponent)*IntegerY/(2^32)32-bit FloatY = (2^Exponent)*IntegerY/(2^16)16-bit |
| 4 | long | Number of points in the file (if not XYXY) |
| 8 | double | First X coordinate |
| 15 | double | Last X coordinate |
| 23 | long | Number of subfiles |
| 27 | byte | X units type code (see SPC.H for values) |
| 28 | byte | Y units type code |
| 29 | byte | Z units type code |
| 30 | byte | Posting disposition (see GRAMSDDE.H) |
| 31 | long | Compressed Date: minutes= 6 bits hour= 5 bits day= 5 bits month= 4 bits year=12 bits |
| 35 | char * 9 | Resolution description text |
| 43 | char * 9 | Source instrument description text |
| 52 | word | Peak point number for interferograms |
| 54 | float * 8 | Spare |
| 86 | char * 130 | Memo |
| 216 | char * 30 | X, Y, and Z custom axis strings (combined) |

| 246 | long | Byte offset to Log Block |
| --- | --- | --- |
| 250 | long | File modification flag (see SPC.H for values) |
| 254 | byte | Processing code (see GRAMSDDE.H) |
| 255 | byte | Calibration level + 1 |
| 256 | word | Sub-method sample injection number |
| 258 | float | Floating data multiplier concentration factor |
| 262 | char * 48 | Method file |
| 310 | float | Z subfile increment for even Z Multifiles |
| 314 | long | Number of W planes |
| 318 | float | W plane increment |
| 322 | byte | W axis units code |
| 324 | char * 187 | Reserved |

### Data Block

The Data Block is the body of the file containing the actual trace data.

- For each subfile, a 32 byte subheader

- All of the X coordinates of the subfiles, if appropriate

- All of the Y values

#### XY Data Block

For XY type single files and XYY Multifiles, the X axis data are stored in a block following the Main Header block and before the Subheader Block as single precision floats.

#### Subheader Block

Each subfile has a 32 byte subheader block. This block exists even for single files.

| Byte # | Data Type | Description |
| --- | --- | --- |
| 1 | byte | Subfile flags. Each bit contains a different parameter: 1 = Subfiles changed 8 = Do not use peak table file 128 = Subfile modified by arithmetic |
| 2 | char | Exponent for subfile's Y values (80h = as floating point) |
| 3 | word | Subfile index number |
| 5 | float | Subfile's starting Z value |
| 9 | float | Subfile's ending Z value |
| 13 | float | Subfile's noise value to use peak picking |
| 17 | long | Number of points if XYXY Multifile |
| 21 | long | Number of co-added scans |
| 25 | float | W axis value |
| 29 | char * 4 | Reserved |

For XY single files and XYY Multifiles, the Y data are stored after each subfile header. They can be stored in one of two formats:

as fixed point integers that are multiplied by the Exponent value

as single precision floating point values (Note: GSPCIO always saves in this format)

For XYXY Multifiles, the data for each subfile is stored with all of the X coordinates (as singles) followed by all of the Y values in one of the above formats.

The SPC file format allows for the handling of 4-dimensional data. This is done using the W axis, which is tracked as follows:

- In the Main header block are parameters specifying the number of W planes, the W increment, and the W axis units.

- The subfile headers then have a parameter setting the W value for that subfile.

- The total number of subfiles are then divided by the number of W planes and consecutive subfiles are "bundled" in each W plane. This means that the total number of subfiles must be an even multiple of the number of W planes.

For example, if there are 5 W planes and 20 subfiles, each block of 4 subfiles will belong to a W plane and will each have the same W value.

#### Directory Block

For XYXY Multifiles, this block contains the offset pointers for each subfile.

### Log Block

The Log Block consists of three parts:

the Log Header block

the binary Log Data block

the ASCII Log Text block

#### Log Header Block

The Log Header is a 64 byte block containing the following fields:

| Byte # | Data Type | Description |
| --- | --- | --- |
| 0 | Long | Size of the Log block (in bytes) |
| 3 | Long | Size of the memory block occupied by the Log block = the Log block size rounded-up to the nearest even multiple of 4096 |
| 7 | Long | Offset to the Text section of the Log block |
| 11 | Long | Size of the Binary Log block |
| 15 | Long | Size of the disk area |
| 19 | Char * 44 | Reserved |

#### Log Data Block

The Binary area of the Log block is used to store data that is required for processing the file, but not displayed. For example, the imaginary data in an NMR file.

#### ASCII Log Text Block

The ASCII Log Text block serves a dual purpose:

- To store acquisition parameters and other information about the file that is necessary for processing.

- To record any changes made to the file by processing programs. This is referred to as the Audit Log.

Main Header

[XYY X Coordinates]

Subfile Header

[XYXY Subfile X Values]

Subfile Y values

..........

Subfile Header

[XYXY Subfile X Values]

Subfile Y Values

[Subfile Directory Block]

Log Block Header

[Binary Log Block]

Audit Log Block

Figure 1. Diagram of SPC File Structure.

# Using GSPCIO

This tutorial gives basic information and tips on using the utility in applications. All of the code examples are in Visual Basic. They assume that suitable variables have been declared and set to valid values. The methods, properties, and events of GSPCIO are described in detail in the Help file GSPCIO.CHM.

Note that index variables for the numbers of points and subfiles should be declared as Long to handle the largest allowed values.

## Defining GSPCIO for Use in Programs

Before using GSPCIO, a variable of its type must be defined in the General Declarations section of the program – either in the Main module if it is global to the project or at the appropriate module level. This is done as follows:

Public SPCIO As New GSpcIOLib.GspcIO

In addition, the object must be added to the References list for the program by selecting: Project / References / Galactic SpcIO Library

The variable SPCIO will be used in all of the following examples. Remember to free the variable when you are done with it or exit the program using:

Set SPCIO = Nothing

## Reading File Properties

### File Type

The basic types of SPC files are: Y-only, XY (for single files) / XYY (for Multifiles) and XYXY Multifiles. The type is determined as follows:

FileType = SPCIO.FileType

GSPCIO will return an integer value for one of the following defined types: 0 = spcFileTypeEven

1 = spcFileTypeXYY

2 = spcFileTypeXYXY

### X and Y Values

The Y values are acquired using the call SPCIO.YPoints. Getting the X values is more complicated since the call used depends on whether the file is Y-only or XY. Therefore, the following code must be used:

pts = SPCIO.NumPoints

If SPCIO.FileType = spcFileTypeEven then FFP = SPCIO.FirstX

FLP = SPCIO.LastX

For i = 0 to pts - 1

X(i) = FFP + i * (FLP – FFP) / (pts-1)

next I else

X = SPCIO.XPoints FFP = X(0)

FLP = X(pts-1)

end if

Note that the X and Y arrays must be dimensioned from 0 to the number of points–1.

### X Axis Limits

The starting and ending X-axis limits of a spectrum are read using the SPCIO.FirstX (= ffp) and the

SPCIO.LastX (= flp) methods.

Note that for XY files, these values do not necessarily correspond to the actual X-axis limits. In this case, you can get the actual limits by reading the file's X array (using SPCIO.XPoints) and reading the values of the 0th and last elements of the X-axis array. For XYXY Multifiles, it is necessary to scan through all of the subfiles and get the outermost X limits (when doing so, be sure to handle the data running from both low to high and high to low).

### Axis Labels

There are two basic types of axis labels in an SPC file: a "standard" label defined by a numerical code in the Main file header, and a "custom" label defined by a string in the Main header block. Because the two values can be set independently, you cannot count on the one not being used to be null. The following code gets the X-axis type code and label:

If SPCIO.XLabel <> SPCIO.XCustomLabel then XType = SPCIO.XType

XCustomLabel ="" Else

XType = 0

XCustomLabel = SPCIO.XCustomLabel End If

### Subfiles

GSPCIO numbers the subfiles in a Multifile from 0 to the number of subfiles –1. Each subfile actually has two Z values: a starting "time" and an ending "time". This was to allow for a scan taking a finite period. The subfiles are accessed as follows:

NumSubs = SPCIO.NumSubfiles For i = 0 to NumSubs-1

SPCIO.SubfileIndex = i ZStart = SPCIO.ZStart ZEnd = SPCIO.ZEnd

Next i

### Audit Log

The Audit Log section of the SPC file serves two functions:

- To record instrument and data acquisition information for use in processing the data

- To provide a record of changes made to the file by processing programs

Because GSPCIO does not have a call to return the number of lines in the Audit Log, you need to acquire the information in a loop until an error condition occurs. When reading the Log text, you need to specify the line number being read.

i = 0

Do While SPCIO.GetLogLine(i, Text(i)) <> -1 LineLen = SPCIO.GetLogLine(i, Text(i))

i = i + 1 Loop

The Log text can also be read-in all at once and then parsed by looking for the <CR><LF> line delimiters. Note that you have to check on both characters because it is possible for them to be in reversed order in the file.

NumChars = SPCIO.LogTextSize LogString = SPCIO.LogText

i = 0

j = 0

Do While i <= NumChars i = i+1

Char = Mid$(LogString,i,1)

If (Char<>vbCr and Char<>vbLf) then Text(j) = Text(j) & Char

Else

j = j+1 End If

Loop

To write to the Audit Log, you just keep adding lines. GSPCIO automatically updates the pointer information in the Log block header.

For i = 0 to NumLines SPCIO.AppendLogLine LogText(i)

Next i

## Creating SPC Files

The code used depends on the type of the file being created. Here, the different file types are presented separately for clarity. In practice, a single code block using SELECT and IF statements to handle all of the types would usually be used. The code samples for Multifiles assume the use of a Recordset (or GdbGrid object) to store the source (single file) spectra. They could just as easily be read from disk files or an array of bytes (also known as a blob).

It should be noted that GSPCIO always writes the Y data in floating point format.

### Y-only Single Files

For Single files, you can just create the file by adding points to the created spectrum. GSPCIO requires that the X array not be passed for Y-only files:

SPCIO.CreateSPC spcFileTypeEven SPCIO.FirstX = FFP

SPCIO.LastX = FLP

SPCIO.AddPoints = Y

### XY Single Files

For XY files, the X axis values must be explicitly passed. To facilitate future use of the file, it is recommended that the First and Last X values be set to the corresponding values in the X axis array.

SPCIO.CreateSPC spcFileTypeXYY SPCIO.FirstX = X(0) SPCIO.LastX = X(pts-1) SPCIO.AddPoints Y,X

### Y-Only Multifiles

To create Multifiles, you successively add each subfile. You must also set the Z order type and Z values. The Z axis type can be one of the following:

ZTypeEven: The subfile Z values are evenly spaced. Only the first Z value and Z Increment are used.

ZTypeOrdered: The subfile Z values are either all ascending or descending, but do not have to be even.

ZType Random: Any Z values are allowed for any subfile.

Note that the subfile numbers are indexed starting at 0. Also, a null placeholder is passed in the X axis field of the GSPCIO call.

SPCIO.CreateSPC spcFileTypeEven SPCIO.FirstX = FFP

SPCIO.LastX = FLP

SPCIO.ZIncrementTypeDefault = ZType ReDim Y(0 to NumPoints-1)

For i = 0 to NumSubfiles-1 RS.AbsolutePosition = i+1 SPCIO.OpenBlob RS.Fields("Trace").Value Y = SPCIO.YPoints

SPCIO.AddSubfile Y, vbNull, ZValues(i), ZValues(i) Next i

### XYY Multifiles

To generate an XYY Multifile, you must explicitly pass the (global) X axis array. In the sample below, the X axis array must be the same for each source file. It is re-read in each pass of the loop for convenience.

SPCIO.CreateSPC spcFileTypeXYY SPCIO.ZIncrementTypeDefault = ZType ReDim Y(0 to NumPoints-1)

ReDim X(0 to NumPoints-1)

For i = 0 to NumSubfiles-1 RS.AbsolutePosition = i+1 SPCIO.OpenBlob RS.Fields("Trace").Value Y = SPCIO.YPoints

X = SPCIO.XPoints

SPCIO.AddSubfile Y, X, ZValues(i), ZValues(i) Next I

SPCIO.FirstX = X(0)

SPCIO.LastX = X(NumPoints-1)

### XYXY Multifiles

For XYXY Multifiles, the X data is different for each subfile, but must always be in the same orientation. This means that the data arrays must be re-dimensioned for each source file. The outermost points must be determined when scanning through the loop. This is incorporated into the file generation loop here, but it could also be made a separate scan through the data.

SPCIO.CreateSPC spcFileTypeXYXY SPCIO.ZIncrementTypeDefault = Ztype

For i = 0 to NumSubfiles-1 RS.AbsolutePosition = i+1 SPCIO.OpenBlob RS.Fields("Trace").Value NumPoints = SPCIO.NumPoints

ReDim Y(0 to NumPoints-1) ReDim X(0 to NumPoints-1) Y = SPCIO.YPoints

X = SPCIO.XPoints

SPCIO.AddSubfile Y, X, ZValues(i), ZValues(i)

If i = 0 Then	'Get outermost X values FFP = SPCIO.X(0)

FLP = SPCIO.X(NumPoints-1)

End If

If X(0) < X(NumPoints-1) Then if X(0)<FFP then FFP = X(0)

if X(NumPoints-1)>FLP then FLP = X(NumPoints-1)

Else

if X(0)>FFP then FFP = X(0)

if X(NumPoints-1)<FLP then FLP = X(NumPoints-1) End If

Next I

SPCIO.FirstX = FFP SPCIO.LastX = FLP

### Disk Files

#### Reading from Disk

GSPCIO can also be used to read an SPC file directly from disk. In this case, it is necessary to validate that the file is in SPC format. An error code is returned if it is not.

On Error Resume Next SPCIO.OpenFile FileName ecode = Err.Number

On Error GoTo 0 Select Case ecode

case 0

MsgBox "This is a valid SPC file. You may proceed." case –1

MsgBox "Unable to open file. File does not exist or" _ & " you do not have rights to it."

case –3

MsgBox "This file is not in SPC format, but is a recognized" _ & " spectrum file that can be converted."

End Select

#### Writing to Disk

Either a single (sub)file or an entire Multifile can be saved to disk:

SPCIO.SaveFile FileName, SubfileIndex

If the subfile index = -1, then the entire Multifile is saved.